# SAFETY DATA SHEET

# Section 1 - PRODUCT AND SUPPLIER IDENTIFICATION

### Product Identification

- Name of the category: naphthas and condensates;

- The name of the product: Natural Gas Condensate;

- Trade name of the product: Rong Doi & Rong Doi Tay Field Condensate;

- Product use: Fuel, solvent.

### Supplier Identification

- Company name: Korea National Oil Corporation Vietnam Office

- Address: 10th Floor, Diamond Plaza,34 Le Duan St., Dis. 1, Ho Chi Minh City, Socialist Rep. of Vietnam

- Telephone: (84-28) 3825-7808

- Fax: (84-28) 3825-7806

- Emergency telephone number: (84-28) 3825-7808

# Section 2 - HAZARDS IDENTIFICATION

### Classification

Physical hazards	Flammable liquids Category 1

Health hazards	Skin corrosion/irritation Category 2

Aspiration Hazard -- Category 1 Skin corrosion/irritation -- Category 2

Acute toxicity, Inhalation -- Category 4

Specific target organ toxicity (single exposure) -- Category 3 Carcinogenicity -- Category 1B

### Label elements

Signal word(s):	DANGER Hazard statement(s):

Extremely flammable liquid and vapor. Causes skin irritation.

May be fatal if swallowed and enters airways. May contain poisonous hydrogen sulfide gas Harmful if inhaled.

May cause drowsiness or dizziness. May cause cancer.

Toxic to aquatic life with long lasting effects.

### Precautionary Statement(s):

Prevention:

Obtain special instructions before use.

Do not handle until all safety precautions have been read and understood. Keep away from heat/sparks/open flames/hot surfaces - No smoking.

Keep container tightly closed. Keep cool.

Ground/bond container and receiving equipment. Use with explosion-proof equipment.

Use only non-sparking tools.

Take precautionary measures against static discharge. Avoid breathing dust/fume/gas/mist/vapors/spray.

Wash thoroughly after handling.

Use only outdoors or in a well-ventilated area. Avoid release to the environment.

Wear protective gloves / protective clothing / eye protection / face protection.

### Response:

IF ON SKIN:	Remove/Take off immediately all contaminated clothing.

Wash with plenty of soap and water.

If skin irritation occurs: Get medical advice/attention. Take off contaminated clothing and wash before reuse.

IF INHALED:	Remove victim to fresh air and keep at rest in a position comfortable for breathing.

Call a POISON CENTER or doctor/physician if you feel unwell.

IF SWALLOWED:  Immediately call a POISON CENTER or doctor/physician.

Do NOT induce vomiting.

In case of fire:	Use dry chemical, carbon dioxide, or foam for extinction.

Collect spillage.

Store in a well-ventilated place. Keep container tightly closed. Dispose of contents/container to approved disposal facility.

Section 3 - COMPOSITION / INFORMATION ON INGREDIENTS

| Components | CAS number | % Mass |
| --- | --- | --- |
| Natural gas condensate (Petroleum) | 64741-47-5 | 100 |
| Benzene | 71-43-2 | 2.3374 |
| Toluene | 108-88-3 | 5.7391 |
| m-Xylene | 108-38-3 | 2.1729 |
| p-Xylene | 106-42-3 | 2.1809 |
| o-Xylene | 95-47-6 | 1.2740 |
| Cyclohexane | 110-82-7 | 2.1220 |
| Ethylbenzene | 100-41-4 | 0.7856 |
| n-Hexane | 110-54-3 | 2.6142 |

# Section 4 - FIRST AID MEASURES

| Inhalation | Immediately remove from further exposure. Get immediate medical assistance. For those providing assistance, avoid exposure to yourself or others. Use adequate respiratory protection. Give supplemental oxygen, if available. If breathing has stopped, assist ventilation with a mechanical device or use mouth-to-mouth resuscitation. |
| --- | --- |
| Skin contact | Immediately remove contaminated clothing. Wash with soap and water. Continue to rinse for at least 15 minutes. In case of rashes, wounds or other skin disorders: Seek medical attention and bring along these instructions. |
| Eye contact | Immediately flush with plenty of water for at least 15 minutes. Get medical attention if irritation develops or persists. |
| Ingestion | Immediately rinse mouth and drink plenty of water or milk. Keep person under observation. Do not induce vomiting. If vomiting occurs, keep head low. Seek immediate medical attention or advice. |
| Most important symptoms/effects, acute and delayed | Headache, drowsiness, dizziness, loss of coordination, disorientation and fatigue Dry skin and possible irritation with repeated or prolonged exposure. |
| Indication of immediate medical attention and special treatment needed | Treat symptomatically. Be aware that symptoms of chemical pneumonia (shortness of breath) may occur several hours after exposure. |

Section 5 - FIRE FIGHTING MEASURES

|  | NFPA 704 Hazard Class Health: 1 Flammability: 4 Instability: 0 (0-Minimal, 1-Slight, 2-Moderate, 3-Serious, 4-Severe) |
| --- | --- |
| EXTINGUISHING MEDIA | Foam. Dry chemical powder. BCF (where regulations permit). Carbon dioxide. Water spray or fog - Large fires only. |
| FIRE FIGHTING | Alert Emergency Responders and tell them location and nature of hazard. May be violently or explosively reactive. Wear breathing apparatus plus protective gloves for fire only. Prevent, by any means available, spillage from entering drains or water course. If safe, switch off electrical equipment until vapor fire hazard removed. Use water delivered as a fine spray to control fire and cool adjacent area. Avoid spraying water onto liquid pools. Do not approach containers suspected to be hot. Cool fire exposed containers with water spray from a protected location. If safe to do so, remove containers from path of fire. Equipment should be thoroughly decontaminated after use. When any large container (including road and rail tankers) is involved in a fire, consider evacuation by 1640 feet in all directions. |
| GENERAL FIRE HAZARDS/HAZARDOUS COMBUSTIBLE PRODUCTS | Liquid and vapor are highly flammable. Severe fire hazard when exposed to heat, flame and/or oxidizers. Vapor forms an explosive mixture with air. Severe explosion hazard, in the form of vapor, when exposed to flame or spark. Vapor may travel a considerable distance to source of ignition. Heating may cause expansion / decomposition with violent rupture of containers. On combustion, may emit toxic fumes of carbon monoxide (CO) |
| FIRE INCOMPATIBILITY | Avoid contamination with oxidizing agents i.e. nitrates, oxidizing acids, chlorine bleaches, pool chlorine etc. as ignition may result |

Section 6 - ACCIDENTAL RELEASE MEASURES

| Personal precautions, protective equipment and emergency procedures | Stay upwind. Eliminate all ignition sources (no smoking, flares, sparks or flames in immediate area). Avoid inhalation of vapors and spray mist and contact with skin and eyes. Wear suitable protective clothing, gloves and eye/face protection. For personal protection, see section 8 of the SDS. |
| --- | --- |
| Methods and materials for containment and cleaning up | Remove sources of ignition. Beware of the explosion danger. Stop the flow of material, if this is without risk. Dike the spilled material, where this is possible. Small Spills: Absorb spillage with non-combustible, absorbent material. Ensure that waste and contaminated materials are collected and removed from the work area as soon as possible in a suitably labeled container. Large Spills: Remove with vacuum trucks or pump to storage/salvage vessels. Use a noncombustible material like vermiculite, sand or earth to soak up the product and place into a container for later disposal. Wash area with soap and water. |
| Environmental precautions | Prevent spreading over a wide area (e.g. by containment or oil barriers). Do not contaminate water. Contact local authorities in case of spillage to drain/aquatic environment. |

Section 7 - HANDLING AND STORAGE

| Precautions for safe handling | Provide adequate ventilation. Avoid inhalation of vapors/mist and contact with skin and eyes. The product is extremely flammable, and explosive vapor/air mixtures may be formed even at normal room temperatures. Ground container and transfer equipment to eliminate static electric sparks. Vapors are heavier than air and may travel along the floor and in the bottom of containers. Use non-sparking hand tools and explosion-proof electrical equipment. Wear appropriate personal protective equipment. Immediately change contaminated clothes. DO NOT EAT, DRINK or SMOKE when using the product. Observe good industrial hygiene practices. Use only bottom loading of tankers, in compliance with European legislation. Do not use compressed air for filling, discharging, or handling operations. Empty containers may contain flammable product residues. |
| --- | --- |
| Conditions for safe storage, including any incompatibilities | Store in a well-ventilated place. Follow rules for flammable liquids. Keep away from heat, spark, open flames and other sources of ignition. Store in a cool, dry place. Store in tightly closed original container. Keep away from food, drink and animal feeding stuffs. Store away from incompatible materials. |

# Section 8 - EXPOSURE CONTROLS / PERSONAL PROTECTION

| Components | CAS number | Occupational Exposure Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | OSHA(1) | NIOSH(2) | VN MOH(3) |  |
| Benzene | 71-43-2 | 1 5STEL | 0.1 1STEL | 1.57 4.7STEL | ppm |
| Toluene | 108-88-3 | 200 300Ceiling | 100 150STEL | 27 80STEL | ppm |
| m-Xylene | 108-38-3 | 100 | 100 150STEL | 23 69STEL | ppm |
| p-Xylene | 106-42-3 | 100 | 100 150STEL | 23 69STEL | ppm |
| o-Xylene | 95-47-6 | 100 | 100 150STEL | 23 69STEL | ppm |
| Cyclohexane | 110-82-7 | 300 | 300 | 145 290STEL | ppm |
| Ethylbenzene | 100-41-4 | 100 | 100 125STEL |  | ppm |
| n-Hexane | 110-54-3 | 500 | 50 | 25 50STEL | ppm |

(1) 8-hour TWA unless otherwise specified.

(2) 10-hour TWA unless otherwise specified

(3) VN MOH: according to Vietnam Ministry of Health STEL: 15-minute Short Term Exposure Limit

Ceiling: Concentration not to be exceeded at any time.

## ENGINEERING CONTROLS

Explosion proof or local exhaust ventilation should be used to: (1) Maintain airborne chemical concentrations below applicable exposure limits, (2) Prevent accumulation of flammable vapors and formation of explosive atmospheres, and (3) Prevent formation of oxygen deficient atmospheres, especially in confined spaces.

## EYE PROTECTION

Eye protection that meets or exceeds ANSI Z.87.1 is recommended if there is a potential for liquid contact to the eyes. Safety glasses equipped with side shields or safety goggles are recommended. Chemical goggles should be worn during transfer operations or when there is a likelihood of misting, splashing or spraying of this material. A face shield may be necessary depending on conditions of use.

## HAND PROTECTION

Avoid skin contact. Use impervious gloves. PVC and neoprene may be suitable for incidental contact. Nitrile rubber should be used for longer term protection when prolonged or frequent contact may occur. Gloves should be worn on clean hands and hands should be washed after removing gloves. Also wash hands with plenty of mild soap and water before eating, drinking, smoking, using toilet facilities or leaving work.

## SKIN AND BODY PROTECTION

Avoid skin contact. Wear long-sleeved fire-retardant garments while working with flammable and combustible liquids. Additional chemical-resistant protective gear may be required if splashing or spraying conditions exist.

## RESPIRATORY PROTECTION

A respiratory protection program that meets or exceeds OSHA 29 CFR 1910.134 and ANSI Z.88.2 should be followed whenever workplace conditions warrant the use of a respirator. Use a full-face positive-pressure supplied air respirator in circumstances where air-purifying respirators may not provide adequate protection or where there may be the potential for airborne exposure above the exposure limits. If exposure concentration is unknown, IDLH conditions exist or there is a potential for exposure to hydrogen sulfide above exposure limits, use a NIOSH approved self-contained breathing apparatus (SCBA) or equivalent operated in a pressure demand or other positive pressure mode.

Section 9 - PHYSICAL AND CHEMICAL PROPERTIES

| Physical state | Liquid |
| --- | --- |
| Color | Yellowish |
| Odor | Gasoline |
| Initial boiling point and boiling range | 30.9oC to 305.5oC |
| Flash point | < -20oC |
| Vapor pressure | 10.15PSIG @ 37.8oC |
| Relative density | 0.7610 kg/L @ 15oC |
| Solubility(ies) | Slightly soluble in water |
| Viscosity | 0.8600 cSt @20oC |

Section 10 - CHEMICAL STABILITY AND REACTIVITY INFORMATION

| Reactivity | The product is stable and non-reactive under normal conditions of use, storage and transport. |
| --- | --- |
| Chemical stability | Stable under normal temperature conditions |
| Possibility of hazardous Reactions | Hazardous polymerization does not occur. |
| Conditions to avoid | Heat, flames and sparks, elevated temperatures. Contact with incompatible materials. |
| Incompatible materials | Strong acids. Strong oxidizing agents. |
| Hazardous decomposition Products | Thermal decomposition or combustion may liberate carbon oxides and other toxic gases or vapors. |

Section 11 - TOXICOLOGICAL INFORMATION

| Components | Hazard | ACUTE TOXICITY DATA |
| --- | --- | --- |
| Benzene | Acute inhalation | IDLH (NIOSH): 500 ppm LC50 (rat): 10,000 ppm / 7hr |
| Toluene | Acute inhalation | IDLH (NIOSH): 500ppm LC50 (rat): 8,000 ppm / 4hr |
| m-Xylene | Acute inhalation | IDLH (NIOSH): 900 ppm LC50 (rat) = 5,984 ppm / 6hr |
| p-Xylene | Acute inhalation | IDLH (NIOSH): 900 ppm LC50 (rat) = 4,550 ppm / 6hr |
| o-Xylene | Acute inhalation | IDLH (NIOSH): 900 ppm |
| Cyclohexane | Acute inhalation | IDLH (NIOSH): 1,300 ppm |

| Components | Hazard | ACUTE TOXICITY DATA |
| --- | --- | --- |
|  |  | LCLo (mice): 70,000 mg/m3 / 2hr |
| Ethylbenzene | Acute inhalation | IDLH (NIOSH): 900 ppm LCLo (rat): 4,000 ppm / 4hr |
| n-Hexane | Acute inhalation | IDLH (NIOSH): 1,100 ppm LC50 (rat): 77,000 ppm / 1hr |

IDLH : Immediately Dangerous to Life or Health Concentrations

Aspiration Hazard: The major health threat of ingestion occurs from the danger of aspiration (breathing) of liquid drops into the lungs, particularly from vomiting. Aspiration may result in chemical pneumonia (fluid in the lungs), severe lung damage, respiratory failure and even death.

Skin Corrosion/Irritation: May cause skin irritation with prolonged or repeated contact. Liquid may be absorbed through the skin in toxic amounts if large areas of skin are exposed repeatedly.

Serious Eye Damage/Irritation: May cause moderate irritation.

Signs and Symptoms: Light hydrocarbon gases are simple asphyxiants and can cause anesthetic effects at high concentrations. Symptoms of overexposure, which are reversible if exposure is stopped, can include shortness of breath, drowsiness, headaches, confusion, decreased coordination, visual disturbances and vomiting. Continued exposure can lead to hypoxia (inadequate oxygen), rapid breathing, cyanosis (bluish discoloration of the skin), numbness of the extremities, unconsciousness and death.

Skin Sensitization: Skin contact is not anticipated.

Respiratory Sensitization: Excessive exposure may cause irritation to the nose, throat, lungs and respiratory tract. Central nervous system (brain) effects may include headache, dizziness, loss of balance and coordination, unconsciousness, coma, respiratory failure, and death.

Specific Target Organ Toxicity (Single Exposure): Not expected to cause organ effects from single exposure.

Specific Target Organ Toxicity (Repeated Exposure): Causes damage to organs (liver, kidneys, blood, nervous system and skin) through prolonged or repeated exposure.

Carcinogenicity: May cause cancer. Exposure to light hydrocarbons in the same boiling range as this product have been associated in animal studies with effects to the central nervous system, peripheral nervous system, liver, and kidneys. The significance of these animal models to predict similar human response is uncertain. Observing good work practices and personal hygiene procedures (Sections 7 and 8) can minimize potential risks to humans.

This material contains benzene. Human health studies indicate that prolonged and/or repeated overexposure to benzene may cause damage to the blood-forming system (particularly bone marrow), and serious blood disorders such as aplastic anemia and leukemia.

Generative Cell Mutagenicity: Some crude oil fractions have been positive in mutagenicity studies.

Reproductive Toxicity: Not expected to cause reproductive toxicity.

### Carcinogenicity:

| Component (CAS No.) | ACGIH(1) | IARC Monographs(2) | US NTP | OSHA Regulated |
| --- | --- | --- | --- | --- |
| Benzene (71-43-2) | A1 | 1 | Yes | Yes |
| Ethyl benzene (100-41-4) | A3 | 2B | No | No |
| Toluene (108-88-3) | A4 | 3 | No | No |
| Xylenes | A4 | 3 | No | No |

(1) ACGIH Carcinogens: A1 = Confirmed human carcinogen, A2 = Suspected human carcinogen, A3 = Confirmed animal carcinogen with unknown relevance to humans, A4 = Not classifiable as a human carcinogen, A5 = Not suspected as a human carcinogen

(2) IARC Monographs: 1 = Carcinogenic to humans, 2A = Probably carcinogenic to humans, 2B = Possibly carcinogenic to humans, 3 = Not classifiable as to carcinogenicity to humans, 4 = Probably not carcinogenic to humans

Section 12 – ECOLOGICAL INFORMATION

Keep out of sewers, drainage areas, and waterways. Report spills and releases, as applicable, under local governmental regulations. May be hazardous to waterways/wildlife.

If released into soil, this product will absorb and may biodegrade in anaerobic conditions. In water it may volatilize.

Section 13 - DISPOSAL CONSIDERATIONS

| Disposal instructions: | Product is suitable for burning in an enclosed, controlled burner for fuel value or disposal by supervised incineration. |
| --- | --- |
| Local disposal regulations | Dispose of in accordance with local regulations for hazardous waste. |
| Waste from residues/ unused products | Dispose of in accordance with local regulations for hazardous waste |
| Contaminated packaging | Since emptied containers may retain product residue, follow label warnings even after container is emptied. |

Section 14 - TRANSPORTATION INFORMATION

## IATA

UN number	UN3295

UN proper shipping name	Hydrocarbons, liquid, n.o.s. (Natural gas condensates)

### Transport hazard class	3

### Packaging group	I

### Labels required	3

### ERG Code	3H

Special precautions for user	Not available.

## IMDG

UN number	UN3295

UN proper shipping name	Hydrocarbons, liquid, n.o.s. (Natural gas condensates)

### Transport hazard class	3

### Packaging group	I

### Labels required	3

EmS	F-E, S-D

Special precautions for user	Not available.

Section 15 - REGULATORY INFORMATION

Decision No.3733/2002/QD-BYT of Ministry of Health: Promulgating 21 labor hygiene standards, 05 principles and 07 labor hygiene measurements: Provides Occupational Exposure Limits

Decree No: 38/2015/ND-CP of Prime Minister: Management of waste

Section 16 - OTHER INFORMATION

Date of Issue	: 25-September-2024 Date of Revision	: 16-August-2024 Rev.		01

### References

ACGIH: Association Advancing Occupational and Environmental Health NIOSH: The National Institute for Occupational Safety and Health

US. IARC Monographs on Occupational Exposures to Chemical Agents Haz-Map® - occupational health database

Disclaimer: The information contained herein is accurate to the best of our knowledge. KNOC makes no warranty of any kind, express or implied, concerning the safe use of this material in your process or in combination with other substances.